enable

conf t

hostname CS1

enable secret cisco

username admin secret cisco

ip domain-name safeBXL.local

crypto key generate rsa

1024

ip ssh version 2

ip routing

vlan 10

vlan 20

vlan 100

vlan 200

spanning-tree mode rapid-pvst

spanning-tree vlan 10,20,100,200 priority 4096

! EtherChannel configuratie

interface range g1/0/2-3

channel-group 1 mode active

exit

interface port-channel 1

switchport mode trunk

exit

! Trunks naar S3 en S4

interface range g1/0/4-5

switchport mode trunk

exit

interface vlan 10

ip address 10.102.0.1 255.255.255.0

standby 10 ip 10.102.0.254

standby 10 priority 110

standby 10 preempt

interface vlan 20

ip address 10.102.1.1 255.255.255.0

standby 20 ip 10.102.1.254

standby 20 priority 110

standby 20 preempt

interface vlan 100

ip address 10.102.2.65 255.255.255.192

standby 100 ip 10.102.2.126

standby 100 priority 110

standby 100 preempt

interface vlan 200

ip address 10.102.2.1 255.255.255.192

standby 200 ip 10.102.2.62

standby 200 priority 110

standby 200 preempt

line con 0

password cisco

login

line vty 0 4

login local

transport input ssh

exit